PURPOSE: The purpose of the procedure is to ensure safe launching of the pig.

SCOPE:      This procedure shall be applicable to all stations of IOCL LPG pipelines.

REFERENCES:

- Operations Manual

- HSE Manual

- Pig Supplier’s Manual

- OISD STD -214

Sprinkler system to disperse LPG vapors to atmosphere

DESCRIPTION:

VENT

VALVE D

Pressure

Gauge

Pig Detector

PP

MOV   A

HOV-A

Closure Door

Cold Flare Unit

Drain Point

2 B 3

X

2 B 1

2 G 1

4 G 1

2 B 2

2 G 2

MOV B

4 B 1

Monitor for Water Spray

(If required)

Kicker Line

LPG Drain Line

SCRAPPER LAUNCHING BARREL

            - Pig tracking device (Pinger) to be fitted in each Bi-Di, Cleaning Pig and Gauge Pig. No pig to be launched without tracking device.

- Ensure that MOV A, HOV-A and HOV 4G-1, 4B-1 and HOV 2G-2, 2B-12 are in close condition.

- Drain the launching barrel by opening drain valves HOV 2B-1 and 2G-1 through Cold Flare unit with continuous water spray from sprinkler nozzles to disperse the LPG vapors to atmosphere.

- Ensure entire LPG is drained out of the barrel (i.e. pressure gauge is indicating Zero pressure), then close the Drain valves HOV 2B-1 and 2G-1.

- Now purge the Launching barrel with Nitrogen (N2) through vent valve D and constantly check the concentration of nitrogen at drain Valve 2B-3.

- Ensure LEL level shall come down to 0 % (at drain Valve 2B-3 outlet) once the entire barrel has been purged with nitrogen.

- When barrel is completely purged with nitrogen, stop the purging process by closing vent valve D & drain Valve 2B-3.

- Open the closure door and insert the pig so the first cup forms a tight fit in the reducer at point X.

- Close the closure door. Again, purge the barrel with Nitrogen as mentioned in step 4 to remove oxygen from barrel before filling LPG into the barrel.

- Check for Oxygen concentration at drain point. It should come down to at least less than 5%.

- After Ensuring oxygen level less than 5% open HOV 4B-1 and 2B-2 partially. Now slowly fill the barrel with LPG through KICKER LINE by opening HOV 4G-1 and HOV 2G-2 simultaneously.

- Ensure barrel is pressured at line pressure and open the HOV 4G-1, 4B-1 and HOV 2G-2, 2B-2 fully.

- Now Open MOV A & HOV-A to through the launching route. The pig is now ready for launch.

- Start closing MOV B slowly till whole flow divert through barrel. This shall increase the flow through Kicker line.

- Ensure that pig leaves the barrel as indicated by Pig alert and enters the mainline.

- After ensuring that Pig left the barrel, first open MOV B fully followed by closing of MOV A, HOV-A & HOV 4G-1, 4B-1 and HOV 2G-2, 2B-2.

- Open the drain valve 2B-1 and 2G-1 to release the pressure in the barrel and bring down the pressure to zero by draining the LPG through Cold Flare Unit with continuous water Spray from sprinkler nozzles to disperse the LPG vapours.

- Note down the exact time of PIG launch to calculate expected time of arrival (ETA) at receiving station along with flow rate, Suction pressure, Discharge Pressure etc.

SAFETY PROTOCOLS:

Following Safety Protocol to be adhered during execution of work.

- Necessary work permit to be obtained from control room for conducting the job.

- Required PPEs in line with PPE Matrix to be ensured during execution of job.

- Job Safety Analysis to be carried out and ensure its compliance during execution of job.

- Toolbox talk to be conducted prior start of job.

Site Engineer / Maintenance engineer or designated site representative must ensure site restoration upon completion of work.

******