PURPOSE:

The purpose of the procedure is to ensure safe receipt and retrieval of pig at the receiving station.

SCOPE:

This procedure shall be applicable to all stations of IOCL’s pipeline except Marine Terminal.

REFERNCES:

- Operations Manual

- HSE Manual

- Pig Supplier’s Manual

- OISD STD - 214

DESCRIPTION:

- Pig tracking device (Pinger) to be installed in each Bi-DI, Cleaning Pig and Gauge Pig before launching. No Pig to be launched without tracking arrangement.

- Ensure pig signals are reset to record pig arrival.

- Check that 10” isolation valve (XV-XX201R) and 16” isolation valve (XV-XX204R) is closed. All drain valves and vents are closed.

- Open bypass valve across 28” valve (XV- XX203R) to pressurize the trap. Keep the vent open so that all the air is replaced by the liquid.

- Close the vents.

- One hour before arrival of the pig direct the flow through trap.

  - Open 28” trap valve (XV-XX203R).

  - Open 16” trap isolation valve (XV-XX204R)

  - Close 24” trap bypass valve (XV-XX202R)

  - Open 10” trap isolation valve (XV-XX201R)

Monitor pig signaler in the trap until the pig arrival is confirmed. Record the receipt time.

- Open 24” trap bypass valve (XV-XX202R) to the fully open position.

- Close 16” trap isolation valve (XV-XX204R)

- Close 10” trap isolation valve (XV-XX201R)

- Close 28” trap valve (XV-XX203R).

- Close 28” trap block valve bypass.

Important: All motorized valves shall be locked out by shutting off junction box power.

Retrieval of Pig:

- Crack open 4” drain vale until the pressure gauge reads zero.

- Close 4” drain valves and monitor pressure gauge to confirm that trap is fully isolated from mainline.

- Open 4” drain valves.

- Monitor the sump level to ensure station does not lock out on high sump level.

- Open vent valve, allowing air to displace liquid.

- Remove the pig carefully without damaging the door “O” ring and door face.

- Wash the pig over the drain tray.

- Clean the trap door and replace “O” ring if any damage is observed.

- Shut the door securely.

- Close 4” drain valve.

- Reset the pig signaler.

- Inform Central Dispatch that pig has been received.

SAFETY PROTOCOLS:

Following Safety Protocol to be adhered during execution of work.

- Necessary work permit to be obtained from control room for conducting the job.

- Required PPEs in line with PPE Matrix to be ensured during execution of job.

- Job Safety Analysis to be carried out and ensure its compliance during execution of job.

- Toolbox talk to be conducted prior start of job.

Site Engineer / Maintenance engineer or designated site representative must ensure site restoration upon completion of work.

******